EI 3643 Human factors standards for human machine interface (HMI) design: quick reference guide

July 2026

First edition

Published by

Energy Institute, London

The Energy Institute is a professional membership body incorporated by Royal Charter 2003

Registered charity number 1097899

EI copyright page

Contents

List of tables and figures

Acknowledgements

Foreword

1 Introduction

  - Human factors engineering in design for safe operations

  - Purpose of this guide

  - HFE integration

  - How to use this guide

- General guidance

  - HMI design guide

  - Structure and navigation

  - Information presentation

  - Menus

  - Legibility and accessibility

    - Visibility at operator eye points

    - Calculating the target size of a visual target

  - Data entry

- Detailed guidance

  - Design features

    - Font Selection and use

    - Colour schemes

    - Icon and symbol design

    - Labels

  - Status and condition monitoring

    - Indicators

    - Scales

    - System messages

    - Alarms and alerts

  - Interaction methods and controls

    - Control types

    - Control design and functionality

    - Control coding

    - Control separation

    - Soft controls

    - Hard (mechanical) controls

      - Discrete rotary controls

      - Continuous adjustment rotary controls

      - Knobs

      - Discrete adjustment thumbwheel controls

      - Cranked handwheel controls

      - Non-cranked handwheel controls

      - Push button controls (finger and hand operated)

      - Toggle switch controls

      - Rocker switch controls

      - Lever controls

Annex A Glossaries

A.1 Acronyms and abbreviations

A.2 Units of measurement

A2.1 Angle

A2.2 Resistance

A.2.3 Torque

Annex B References

List of figures and tables

Tables

Table 1: Design principles for structure and navigation features

Table 2: Design principles for the presentation of information

Table 3: Guidance on menu presentation

Table 4: Design principles for viewing distance to human machine interface (HMI) targets

Table 5: Scale markings: visual angle for dark markers on a light background

Table 6: Example - calculating the preferred size of the visual target to achieve arc angle

Table 7: Design principles for data entry

Table 8: Design principles for fonts

Table 9: Design principles for the use of colour

Table 10: Design principles for icons and symbols

Table 11: Design principles for labels

Table 12: Design principles for indicators

Table 13: Simple indicator lights:  size and colour coding

Table 14: Design principles for scales

Table 15: Design principles for messaging

Table 16: Control type definitions

Table 17: Design principles applicable to mechanical and soft controls

Table 18: Control coding:  advantages of different coding styles

Table 19: Minimum edge to edge separation distances for mechanical controls

Table 20: Separation distances under different operation conditions for mechanical controls

Table 21: Design principles for soft controls

Table 22: Dimensions and resistance for discrete rotary controls

Table 23: Dimensions and resistance for continuous adjustment thumbwheel controls

Table 24: Dimensions for knobs

Table 25: Torque for knobs

Table 26: Dimensions and resistance for discrete adjustment thumbwheel con

Table 27: Handle dimensions and turning radii for cranked handwheel controls

Table 28: Configurations, applications and dimensions for non-cranked handwheel controls

Table 29: Dimensions and resistance for push button controls

Table 30: Dimensions and resistance for toggle switch controls

Table 31: Dimensions and resistance for rocker switch controls

Table 32: Diameters and resistance for lever controls

Figures

Figure 1: Example HMI structure with zones

Figure 2: Example HMI structure

Figure 3: Example HMI structure with menu

Figure 4: Arc angle at operator’s eye point to an HMI target

Figure 5: Scale markings: visual angle for dark markers on a light background

Figure 6: Features of visual angle

Figure 7: Example HMI structure with colour scheme

Figure 8: Example of icon design

Figure 9: Example of label design

Figure 10: Example of status indicators

Figure 11: Continuous rotary controls in orientation A

Figure 12: Continuous rotary controls in orientation B

Figure 13: Discrete rotary control

Figure 14: Discrete rotary control dimensions and resistance

Figure 15: Examples of continuous adjustment rotary controls

Figure 16: Dimensions and resistance for continuous adjustment thumbwheel controls

Figure 17: Example of a rotary knob control

Figure 18: Rotary knob control dimensions for fingertip grasp

Figure 19: Rotary knob control dimensions for thumb and finger encircled grasp

Figure 20: Rotary knob control dimensions for palm grasp

Figure 21: Example of a discrete adjustment thumbwheel control

Figure 22: Dimensions and resistance for discrete adjustment thumbwheel controls

Figure 23: Example of a cranked handwheel control

Figure 24: Handle dimensions and turning radii for cranked handwheel controls

Figure 25: Example of a non-cranked handwheel control

Figure 26: Placement in relation to a display reference

Figure 27: Placement as a locking/unlocking mechanism

Figure 28: High torque valves in various positions

Figure 29: Example of a push button

Figure 30: Push button diameter

Figure 31: Example of a toggle switch control

Figure 32: Dimensions of a toggle switch

Figure 33: Example of a rocker switch control

Figure 34: Dimensions for rocker switch controls

Figure 35: Example of a lever control

Figure 36: Diameters and operational directions for lever controls

# ACKNOWLEDGMENTS

EI 3643 Human factors standards for human machine interface (HMI) design: quick reference guide was authored by Fiona Bird, Ruth Parsons and Ben Smith (Pendyne) and produced by the EI Human and Organisational Factors Committee (HOFCOM).  At time of publication, HOFCOM members included:

Dr Curt Braun				BP plc

Jonathan Bohm 			UK Health Security Agency

Liz Butterworth 				EET Fuels

Christophe Catala 			Rio Tinto

Diane Chadwick-Jones

Jenny Cooper                                          RAS Ltd.

Ed Corbett 				ONR

Alan Dillien				National Grid

Dr Dino DiMattia			ExxonMobil Corporation

Adam Goodman			Phillips 66 (Chair)

Jamie Harcus				Harbour Energy

Zamina Hayes				Shell

Stuart King 				EI (Secretary)

Neal Lawis				PRAX

Mary Marshall				Health and Safety Executive

Richard Marshall			HRA

Oliver Mellors				ACWA Power

Simon Monnington 			Mondelēz International, Inc.

Garry Moon				Health and Safety Executive

Alexandros Paschalis			EI

Hector Silva				Chevron

Gillian Vaughan 			EDF

Frank Verschueren			Former process safety inspector

Project management of this project and technical editing were carried out by Stuart King (EI).

# FOREWORD

The integration of human factors engineering (HFE) good practice into human machine interface (HMI) design is critical to enable operators to conduct their tasks efficiently and, most importantly, to reduce the risk of errors that can lead to catastrophic outcomes. Rather than blaming the operator, it is critical to integrate HFE good practice early in the design life cycle and use the extensive knowledge that is available in existing standards to inform the design and mitigate risks.

This publication has been developed to summarise key design requirements and considerations in relation to HMI design, as described by commonly used human factors design standards and industry good practice.  It is intended for use by specialists from procurement, HFE, design and approvals.

This publication offers both general and detailed design guidance. This guidance has been extracted from applicable standards and summarised into this document as design guidance for those supporting the design of both virtual and hard control HMIs.

The guidance in this publication is designed to offer quick reference material that should inform all stages of the design. While it is understood that the full standards should be referenced as part of the design process, this document will provide a quick reference guide to allow design decisions to be made that align with applicable standards.

Whilst the EI and the contributors have applied reasonable care in developing this publication, no representations or warranties, expressed or implied, are made by the EI or any of the contributors concerning the applicability, suitability, accuracy or completeness of the information contained herein and the EI and the contributors accept no responsibility whatsoever for the use of this information.

Neither the EI nor any of the contributors shall be liable in any way for any liability, loss, cost, or damage incurred as a result of the use of the information contained herein.

The EI welcomes feedback on its publications. Feedback or suggestions should be submitted to:

Technical Department, Energy Institute

61 New Cavendish Street, London, W1G 7AR

e: technical@energyinst.org

# INTRODUCTION

## Human factors engineering in design for safe operations

HFE is a multi-disciplinary science that focuses on the interaction between the human and the work system in order to design human-machine interactions that optimise human and system performance and reduce the risk of human error and subsequent consequences (Health and Safety Executive (HSE), website, Control room design).

A human machine interface (HMI) is the means by which a human interacts with a system, process, or machine, typically including visual displays, controls, indicators, and feedback mechanisms. In the energy sector, HMIs are critical for the safe and efficient monitoring and controlling of plant operations.

One of the main purposes of HFE in the design of HMIs is to improve the efficiency of operational, maintenance and emergency task performance. In addition, the application of HFE principles and good practice can reduce the likelihood of human error contributing to a process safety failure, loss of production, environmental release, or major accident.

## Purpose of this guide

The purpose of this HFE quick reference guide is to consolidate and summarise good practice from available industry standards, supplemented with the authors’ experience of HFE industry practice (industry practice, IP).  It aims to provide a single source of information that enables the document user(s) to have confidence that new or modified HMIs will comply with reputable standards and reduce the likelihood of HFE risks.

The document is intended for use by:

- Designers of HMIs to improve the usability of their products and reduce the risk of human error and its associated consequences.

- Those who specify and procure HMIs to ensure that HFE requirements are included in any invitations to tender, bid proposals or off the shelf solution packages.

- HFE Specialists (or those responsible for accepting and approving an HMI for operational use) as a good practice checklist for the acceptance of HMIs.

The guidance is applicable to any location where the system requires a human to perform a function via an HMI to enable the system to operate; this may include:

- HMIs that include control devices, panels, and consoles (soft or hard);

- single HMIs for discrete/standalone systems, e.g. an emergency shutdown panel;

- HMI(s) for distributed/complex control system(s), e.g. an energy management system, and

- alarm and warning systems.

The document is not intended to present all design guidance available in the standards; rather the purpose is to provide the most common HFE good practice in a form that is accessible and easy to implement. It is acknowledged that the source standards were developed for specific domains and therefore guidance may differ between them.

## HFE integration

This guide will not mitigate the risk of human error (or the consequences) in its entirety.

The probability of design-induced human errors in operating and maintenance can be significantly reduced by implementing HFE requirements from the beginning of the system concept/selection phase.  As such, this document is to be used as an input to a wider HFE integration programme -  EI 454 Human factors engineering in projects and ISA-101.01-2015 Human machine interfaces for process automation systems provide guidance for this process.

The design of the HMI should be developed with input from the operators who will be using it, throughout the lifecycle.  This should include design reviews and testing throughout the lifecycle, considering all modes of operation, operational scenarios and the operational environment in which it will be used (IEC 63303).

The user of this document should ensure that the correct guidance is utilised for the specific application.  As such, this document also provides references to more detailed standards that should be consulted for further detail or application-specific guidance.

## How to use this guide

Where possible, a pragmatic approach to the document should be applied. The document user should select the most relevant sections for their use case. Where requirements might contradict each other, it is up to the document user to decide on their preferred approach or utilise the source standard to find more detailed information.

An effort has been made to organise the content into logical sections to support clarity and ease of use. However, some guidance principles are inherently related and may be relevant to more than one section. For example, aspects of soft controls may also relate to system feedback and other interaction elements. Users of this document should therefore consider the guidance holistically.

Where requirements from standards are summarised, these are generally worded in a factual way, for example ‘X is done’, and the text is supported by illustrations and/or data.

The illustrations used within this publication are designed to provide design guidance and are not meant to be proscriptive. Depending on the type of HMI, there are multiple different design layouts that can be used that would conform to the general requirements.

The measurements provided are most appropriate for United States (US) and European populations but may be applicable more broadly. Where there is a need for a greater clarity, more directly applicable standards should be consulted.

# GENERAL GUIDANCE

This section outlines high-level design principles to guide the development of the HMI.

## HMI design guide

It is good practice to define the HMI and develop a dedicated HMI design guide (IEC 63303) describing how operator environment, scenarios and tasks inform the user interactions with the HMI and therefore the presentation of information on the display. This can be informed by activities such as task analysis (IEC 63303) or day in the life of (DITLO) scenarios (IP).

The guide should define the approach to functional allocation, HMI layout, navigation map, the prioritisation and prominence of information based on criticality, and the consistent use of design elements such as fonts, colour schemes, icons, labelling, and unit of measurement (IEC 63303, IP).

When developing the guide there is a need to balance familiar interface conventions and novel design approaches to ensure consistency and usability while minimising the risk of operator confusion or incorrect mental models.

Whilst familiar displays may not adhere to HFE good practice, their familiarity to operators can mitigate risk.  However, it is critical to question the mindset of ‘we have always done it this way’ to avoid accepting poor design and inherent risk.  This is a particular challenge during the integration of existing (or legacy) systems with new interfaces.

As such, it is critical to involve operators in the development of the design guide to ensure that the HMI is conveying the correct meaning (IEC 63303).

## Structure and navigation

The structure and navigation of the interface is planned in the early design phase. This is critical to enable operators to locate and access required information efficiently, reducing the risk of delay, disorientation, or navigation errors during normal and abnormal operations.

Table 1 provides design principles for the development of a structure and navigation map.

Table 1: Design principles for structure and navigation features

| # | Design principles | Ref |
| --- | --- | --- |
|  | Layout: - A standard display screen organisation is evident when locating various HMI functions (such as a data display zone, control zone, or message zone) from one display to another - Organisation of the display is consistent across functions, plant and installations - Information is not arranged on the display in a way that could confuse the operator - The display viewing area is large enough to display all important information rather than being split across displays or requiring excessive scrolling - Sufficient display area is provided to ensure that short-term control tasks can be performed without interfering with longer-term ones | NUREG-0700 BS EN 61227:2016 IEC 63303 |
|  | Navigating structures: - The ordering and layout of corresponding data fields across displays is consistent from one display to another - The layout of information should reflect the sequence, if any, in which it is used - Displays used in sequence are navigable with a single action (e.g. a previous/next button, without need to navigate via a menu) - Pages at the same hierarchy level can be navigated laterally, without returning to the top menu | NUREG-0700 BS IEC 61772:2013 IEC 63303 |
|  | Navigation cues and controls: - Each display page has a unique identifier, reference or title - Users are supported to navigate top-down or bottom-up - A backtrack or undo tool is provided to the operator in a consistent format across all displays - The operator can easily return to the default view from a zoom or scroll view - The operator can cancel an action or sequence before it is completed if they notice an error | NUREG-0700 BS IEC 61772:2013 |
|  | Feedback: - Users are informed when common navigational paths diverge towards different outcomes, e.g.: - ‘This action cannot be undone’ - ‘Feeder 3 supplies critical load. Opening CB-204 will disconnect power to Industrial Bus B’ | NUREG-0700 |

Figure 1 provides an example of structure and navigation features.

Figure 1: Example HMI structure with zones

## Information presentation

The design and presentation of information on HMIs is planned to ensure operators can accurately perceive, interpret, and prioritise critical information.  The careful presentation of information can reduce the risk of misinterpretation or missed cues, improving the reliability of the overall system.

Table 2 provides design principles on the presentation of information. Figure 2 provides an example with functional grouping.

Table 2: Design principles for the presentation of information

| # | Design principles | Ref |
| --- | --- | --- |
|  | General design: - Features on the display directly represent physical functions and states in the plant - Commands are assigned consistently to the same function across different modes, displays and systems with which the operator interacts - All terms employed in the operator-system interface, and their abbreviations, are consistent in meaning from one task to another, and with job aids and documentation - A standard format for data, labels, and other information is used across all displays - Vocabulary, symbols, icons and colour schemes are meaningful and familiar to the operator and consistent across the displays - Wording of data, labels, and other information is typical for the task being performed, and unfamiliar design or programming jargon - i.e., terminology taken from unrelated text - is not used - User-added text is distinguishable from other text - Units of measurement are sufficient for accuracy - Multi-page lists are avoided. However, where used, the last line on a page is repeated at the top of the next page and lists are partitioned logically into shorter lists - Mixed case (upper and lower case) rather than all capital letters is used, except for abbreviations and acronyms | NUREG-0700 BS EN ISO 9241-13:1998 NASA-STD-3001 BS IEC 61772:2013 IP |
|  | Status information: - Multi-modal displays clearly indicate the current mode, and any change of mode - Graphic changes unambiguously represent the physical state of the associated system function - Visual indication is provided to operators when functions or data parameters are unavailable - Visual indication is given to remind operators of interrupted tasks | NUREG-0700 BS EN ISO 9241-13:1998 NASA-STD-3001 |
|  | Functional grouping: - Functions are organised in groups or layers - Items on multi-page displays, or across multiple panels, are grouped by function or task - Controls are grouped around related functions, purposes, or sequences - Items are displayed in a way that reflects their relationships within the plant/installation process, without compromising screen functionality and clarity, e.g. through borders, colours or titles | NUREG-0700 NASA-STD-3001 BS IEC 61772:2013 |
|  | Help functions: - Users can choose when to receive help and guidance, and to suppress unwanted help - An online help section is always quickly accessible or automatically activated by the system, particularly if operators may be inexperienced or use the system (or function) infrequently - Help topics can be searched, are structured hierarchically, or are mapped - After accessing the help function, the operator can easily return to the task | NUREG-0700 BS EN ISO 9241-13:1998 |

Figure 2: Example HMI structure

## Menus

Menus provide a structured way for operators to navigate the interface and access system functions.  Effective menu design and presentation support efficient task execution and selection accuracy, reducing the risk of incorrect selections or delays in accessing functions.

Table 3 provides design principles on the presentation of features using menus, and an example is provided in Figure 3.

Table 3: Guidance on menu presentation

| # | Design principles | Ref |
| --- | --- | --- |
|  | Location and type: - Menus are in the same place on the screen across modes and functions - Menus do not obstruct the view of other items on the page. - Users can see the full menu structure and where the current display sits on that structure - User-requested (pull down or pop out) menus are used in preference to permanent menus, unless the menu is in continual or frequent use | NUREG-0700 BS EN ISO 9241-13:1998 |
|  | Content: - Menus have explanatory titles and the function is clear - Menu options are worded as commands to the computer rather than questions to the operator - Menus display all essential options for the current task, and no more - Currently unavailable options are not shown to the operator or are differentiated in appearance - Options on long menus are grouped logically by function; if this is not possible, they are alphabetical - All options are visible without scrolling: - Long menus are split up into a hierarchical sequence - Options are always presented in the same order | NUREG-0700 BS EN ISO 9241-13:1998 |
|  | Selection: - Accessing functions or returning to a previous menu does not require an excessive number of steps through hierarchical menus - Available menu items are highlighted when the cursor passes over them - ‘On’ and ‘off’ function menu items have different appearances in the two states, the ‘on’ (or more safety-critical) state is more prominent - Menu items remain highlighted once selected - Once an item has been selected, the menu returns to its hidden state | NUREG-0700 BS EN ISO 9241-13:1998 |
|  | Shortcuts: - If keyboard shortcuts are displayed on menu labels: - Key codes are distinctive, based on the name of the option, and used consistently across functions - Customisable codes are easily reset to defaults - Key shortcuts for menu options are mapped on the keyboard to match the order on the menu list | NUREG-0700 BS EN ISO 9241-13:1998 |

Figure 3: Example HMI structure with menu

## Legibility and accessibility

Application of the viewing distance to the HMI (see Figure 4) ensures that information remains visible and usable at the operator’s seated or standing eye point under all environmental and lighting conditions.  This reduces the risk of the operator misreading or overlooking critical information.

Figure 4: Arc angle at operator’s eye point to an HMI target (determining the apparent size and readability of text and other information when considering 95th percentile male and 5th percentile female eye heights when sitting or standing)

The design of workstations is outside the scope of this standard and detailed guidance on workstation layout and position of displays can be found in ISO 11064 Ergonomic design of control centres, EEMUA 201 Control rooms: A guide to their specification, design, commissioning and operation and IEC 63303 Human machine interfaces for process automation systems.  However, dimensional elements of the workstation, such as user eye height, viewing distance, field of vision, cone of fixations and normal line-of-sight are needed to complete visibility calculations.

### Visibility at operator eye points

Table 4 provides recommended guidance on the visual angle (minutes of arc) that characters should subtend at the operator’s eye to ensure they are legible.  The calculation method is provided in 2.5.2.

The suitability of target sizes (and readability) is confirmed through testing with operators in environments that represent the actual conditions in which the system will be used (IEC 63303).

Table 4: Design principles for viewing distance to HMI targets

| # | Design principles | Ref |
| --- | --- | --- |
|  | General principles: - Viewing distance is large enough to minimise eye strain and improve depth of focus. All detail is visible at the maximum viewing distance - Contrast, font, colour and size of text should enable legibility at the expected viewing distance | BS EN ISO 11064-4 BS EN ISO 9241 – Part 303 BS EN ISO 11064-5 |
|  | Characters: - Monochrome Latin characters are recommended to: - Subtend a minimum of 15 minutes of arc (in accordance with ISO 9355-2) - Subtend an optimal range of 18 to 22 minutes of arc - Subtend a preferred range of 25 to 35 minutes of arc - Japanese characters are recommended to: - Subtend a minimum of 20 minutes of arc - Subtend an optimal range of 25 to 35 minutes of arc - When a displayed symbol of complex shape is to be distinguished from another symbol shape that is also complex, the symbol subtends not less than 20 minutes of arc | BS EN ISO 11064-4 BS EN ISO 9241 – Part 303 ISO 9355-2 |
|  | Colours: - Where accurate colour identification of strings of characters or symbols is required, their minimum size is at least 20 minutes of arc - Where accurate colour identification of an isolated image (including characters or symbols) is required, the image subtends at least 30 minutes of arc, preferably 45 minutes of arc | BS EN ISO 9241 – Part 303 |

Figure 5 and Table 5 suggests visual angles for the size of dark scale markings on light background.

Figure 5: Scale markings: visual angle for dark markers on a light background (see Table 5 for key)

Table 5: Scale markings: visual angle for dark markers on a light background (MIL-STD-1472H)

| Scale markings: visual angle |  |  |  |
| --- | --- | --- | --- |
| A | Width of major scale index | 1,16 mrad | 4 min* |
| B | Width of minor scale index | 0,87 mrad | 3 min* |
| C | Width of intermediate scale index | 1,16 mrad | 4 min* |
| D | Length of major scale index | 7,86 mrad | 27 min |
| E | Length of minor scale index | 3,49 mrad | 12 min |
| F | Length of intermediate scale index | 5,82 mrad | 20 min |
| G | Width of gap between major scale index | 25,02 mrad | 86 min |
| H | Width of gap between minor scale index | 2,62 mrad | 9 min |
| NOTES: For most applications with a dark graduation mark on a light background, the width of the minor graduation mark can be used for major and intermediate graduation marks as well. Use of this strategy allows the width of the pointer tip to be the same as all the graduation marks. Visual angles are for the longest anticipated viewing distance. min = minutes of arc mrad = milliradian *4,36 mrad (15 min) for light markers on dark background. |  |  |  |

### Calculating the target size of a visual target

A visual target on the display could be alphanumeric characters, icons, symbols or other critical information.

The angle of arc at the operators’ eye point, or the preferred size for a visual target on the display, can be calculated using trigonometry.  Figure 6 illustrates the features for determining the visual angle and Table 6 provides the calculation and a worked example of how to determine the preferred size of a target.

Figure 6: Features of visual angle (adapted from BS EN ISO 11064-4)

Table 6: Example - calculating the preferred size of the visual target to achieve arc angle

| Step | Description | Example |
| --- | --- | --- |
|  | Determine viewing distance Measure the maximum distance (D) from the operator's eye to the display | D = 1,5 m |
|  | Choose the required visual angle and convert to degrees Select a value between 15 and 45 minutes of arc depending on criticality of target | σ = 16 minutes of arc Convert to degrees by dividing by 60: 16′ = 0,2667° |
|  | Calculate the height that the character will need to be to achieve the required visual angle, using the formula: h=2D×tan (σ/2) | h=2(1,5) tan (0,2667/2) h≈0,0070 m h≈7 mm |

## Data entry

Data entry is the process by which operators input data into a system through devices such as keyboards, touchscreens or control panels. HMI design should minimise error risk and cognitive workload during data entry.

Data entry methods and interfaces should enable operators to enter information accurately and efficiently, reducing the risk of incorrect, incomplete or unintended inputs. This is particularly important for systems where decisions depend on the accuracy of the data entered.

Table 7 provides the design principles for data entry.

Table 7: Design principles for data entry

| # | Design principles | Ref |
| --- | --- | --- |
|  | Location: - Data entry fields are in a consistent location across pages | NUREG-0700 |
|  | Navigation: - Data entry forms can be completed using a minimum number of simple actions and the operator does not need to navigate between pages, particularly when a fast response is required | NUREG-0700 |
|  | Field functionality: - The operator is not required to enter information unnecessarily (e.g. when it is already available to the system) - If there is a default entry value, this is visible to the operator. Guidance is provided on required format and acceptable values for data entries - The operator is not required to remember special codes or sequences, or to perform translations or conversions before entering data - Previous commands that will change the outcome of an entry are displayed on the data entry screen - Common spelling errors will not cause the system to run the incorrect command - The incorrect entry remains visible to the operator and the cursor is positioned over it by the system - The operator can make simple edits without using a specific editing mode - If data entry forms have a hardcopy equivalent, the format is the same - Data entry fields and labels are spaced sufficiently for the operator to read all information - Required and optional data entry fields are distinguished. - Data entry methods can be customised to suit operators with different levels of experience - The operator is not required to enter dashes or slashes to separate data entries. They are not required to enter units of measure (e.g. mm) - Data entry does not overwrite characters in a field - Data entry is not accomplished through speech unless other methods (e.g. keying) are not suitable | NUREG-0700 |
|  | Feedback: - When the operator makes an invalid data entry, or when data entered does not match the predefined format, the operator receives an auditory alert - Feedback is received on all data entries, either an error message or confirmation of success | NUREG-0700 |

# DETAILED DESIGN GUIDANCE

## Design features

### Font Selection and use

The selection and use of fonts within the HMI will ensure text is clear, legible, and easily distinguishable under operational conditions, reducing the risk of misreading or overlooking critical information.

Table 8 provides design principles for the development and implementation of fonts and associated formatting or typographic emphasis.

Table 8: Design principles for fonts

| # | Design principles | Ref |
| --- | --- | --- |
|  | Font selection: - Clear, simple sans-serif fonts are selected - Decorative, condensed, or overly stylised fonts are avoided - Alphanumeric styles are simple and consistent - Font size is consistent across related sections of text, and between related tables (see also 4.5 for legibility) | NUREG-0700 BS EN ISO 9241 – Part 303 IEC 63303 |
|  | Font size: - If the task requires a constant text size, each character does not vary by more than 5% of its height - For Latin-origin characters, the stroke width is within the range of 10% to 17% of character height - A minimum of two stroke widths, or 15 % of character height, (whichever is greater) of space is consistently left between lines of text | NUREG-0700 BS EN ISO 9241 – Part 303 |
|  | Width to height ratios (W:H): - A 4 x 5 (W:H) character matrix is the minimum used for subscripts and superscripts, and for numerators and denominators of fractions displayed in a single character position. The 4 x 5 matrix may also be used for alphanumeric information not related to the operator’s task, such as copyright information. A 7 x 9 (W:H) character matrix is the minimum used for tasks that require continuous reading for context, or where individual alphabetic character legibility is important for the task, such as proofreading. A 5 x 7 (W:H) character matrix is the minimum used for numeric and upper case-only presentations | BS EN ISO 9241 – Part 303 IEC 63303 IP |
|  | Formatting: - Information is highlighted through use of a different font style rather than by different font sizes - Do not use bold, italics, or underlining to show selection points (i.e. buttons, menu items) if these styles are also used in the text for emphasis, as this may cause confusion | NUREG-0700 |
|  | Spacing: - The minimum number of pixels between words is the same number of pixels as within the width of an unaccented upper case letter H. Letter N is used for proportionally spaced fonts - Consistent spacing between the words of displayed text is maintained, with left justification of lines and ragged right margins | NUREG-0700 BS EN ISO 9241 – Part 303 |
|  | Numbers: - Arabic rather than Roman numerals are used in numbered lists - Numeric values are displayed using the decimal system rather than as fractions - Leading zeros are suppressed for whole numbers (e.g. 28 rather than 0028) - A leading zero is used if the number is a decimal with no proceeding integer (e.g. 0.43 rather than .43) - The number of digits presented to the operator is not greater or fewer than what is required to complete the task (e.g. temperature reading of 27.5°C rather than 27.4999999°C) | NUREG-0700 |

### Colour schemes

The appropriate use of colour within the interface supports clear communication of status and priority while reducing the risk of misinterpretation, ambiguity, or failure to recognise important conditions.

Table 9 provides the design principles for the use of colour and an example is provided in Figure 7.

Table 9: Design principles for the use of colour

| # | Design principles | Ref |
| --- | --- | --- |
|  | High level principles for the development of a colour scheme are: - Colour coding follows the conventions of other systems that operators are familiar with - Colour coding aligns with local custom and cultural norms (e.g. red for danger in Western cultures) - Use colour to highlight abnormal conditions, alarms, or information requiring operator action - Use grey-scale colours to represent equipment and process conditions during normal operation - The number of colours used is kept to a minimum to avoid the use of visual clutter - Colours are not used casually without meaning. Each colour is assigned one meaning - Colour coding is used consistently | NUREG-0700 BS IEC 61772:2013 BS EN ISO 9241 – Part 303 IEC 63303 |
|  | Background colours: - Background colour is uniform and allows the data to be easily visible, i.e. it provides sufficient contrast - Displayed information is provided in a contrasting colour to the background - Dark coloured characters are presented on a bright background and light on a dark background | NUREG-0700 BS EN ISO 9241 – Part 303 |
|  | Colour for categorisation, prioritisation and comparison: - High colour saturation is used for important information and low saturation for less important information and design features (e.g. lines) - Information that must be compared or integrated is presented in similar colours - Different categories of information are distinguished, e.g. through colour coding - Colour coding is used to indicate operating ranges or abnormal readings on scales | NUREG-0700 BS IEC 61772:2013 |
|  | Colours for alarms and alerts: - Visual alarms and alerts are colour coded according to priority level; for example: - High priority alarms are presented in a bright colour - Lower priority alerts are presented in a more muted colour | EEMUA 191 |
|  | Colour vision deficiency (CVD): - Unless all operators have been screened for CVD, it is possible to complete tasks safely without full colour vision - Safety critical colour coding is used in conjunction with other forms of coding (e.g. position or shape) rather than as the only cue | BS IEC 61772:2013 NASA-STD-3001 |

Figure 7: Example HMI structure with colour scheme

### Icon and symbol design

This section specifies requirements for the design and use of icons (graphical representations of functions or actions) and symbols (standardised graphical signs representing system states, hazards or conditions) to ensure their meaning is clear and consistently interpreted, and elicits the correct response.

Table 10 summarises the design principles for icon and symbol design, and examples are provided in Figure 8.

Table 10: Design principles for icons and symbols

| # | Design principle | Ref |
| --- | --- | --- |
|  | General principles: - Shape coding follows established standards or conventional meanings - Symbols and words are not used alternately - Definitions for all icons and symbols are available to the operator in a dictionary or online help - Icons have text labels located on the item or close to it; a line between the label and icon may be used for clarity | NUREG-0700 |
|  | The shape of the icon or symbol: - Is representative of a single object or action - Looks like the object, process, or operation they represent or conform to operator conventions - Is capable of only one logical interpretation of meaning - Is standardised across displays and systems - Is easily distinguished. | NUREG-0700 NASA-STD-3001 BS IEC 61772:2013 BS EN ISO 9241 – Part 303 |
|  | The design of the icon or symbol: - Is a simple, closed figure (i.e. with a continuous outside border) - Is oriented upright | NUREG-0700 |
|  | Size of the icon or symbol: - Is large enough for operators to identify and distinguish them - Is legible at different zoom levels - Size coding of icons is not used if the display is crowded. A small range of sizes is used so the operator can distinguish each size | NUREG-0700 BS IEC 61772:2013 |

Figure 8: Example of icon design

### Labels

Clear and consistent labelling is essential to ensure that operators can quickly identify equipment, controls and information presented within the HMI. Well-designed labels support efficient operation, reduce the likelihood of misinterpretation, and help operators understand the function and status of interface elements.

Table 11 provides design principles for labels and an example is provided in Figure 9.

Table 11: Design principles for labels

| # | Design principle | Ref |
| --- | --- | --- |
|  | Application: - Labels are consistent with the labelling plan and icon library as established for a particular HMI design/use case - Items are labelled to assist operators to correctly identify their function and meaning - Labels for a specific graphical object are placed near the object - Adequate and uniform spacing is provided between labels and other interface elements to maintain readability - Labels should use mixed case (upper and lower case) rather than all capital letters, except for abbreviations and acronyms | NASA-STD-3001 IP |
|  | Content: - Labels for tables of data include the unit of measurement used - If icons are used for control actions, they are labelled to clearly show what action they represent | NUREG-0700 |
|  | Design: - Labels have a minimum font height of 12-point or 0,4 degrees in expected operating positions - Labels are distinguished (e.g. by highlighting or capitalising) to differentiate them from other text - If labels are clustered, critical items have distinctive borders | NASA-STD-3001 NUREG-0700 |

Figure 9: Example of label design

## Status and condition monitoring

System status and operating conditions (i.e. system state, performance, and abnormal conditions) are presented to the operator using indicators, scales, messaging and terminology, and supported by appropriate alarms and alerts. Effective presentation of this information supports timely operator awareness, understanding, and response.

Figure 10 illustrates the status indicator information described in 3.2.1 – 3.2.3.

Figure 10: Example of status indicators

### Indicators

The design and presentation of visual indicators ensures that system states, visual alerts, and changes in condition are clearly and reliably communicated, reducing the risk that operators fail to detect, recognise or correctly interpret critical information.

Table 12 provides high level design principles for the design of indicators and an example is provided in Figure 10.

Table 12: Design principles for indicators

| # | Design principles | Ref |
| --- | --- | --- |
|  | Prioritisation: - Indicators that are most important to the operator’s tasks are most prominent in view - The operator’s attention is drawn by coding or highlighting of information to important changes to conditions in the plant - Highlighting of information is removed when the situation is resolved - Unacknowledged and acknowledged indications are visually distinct | NUREG-0700 BS EN ISO 11064-4 BS EN ISO 11064-5 |
|  | Status: - The operator can clearly tell whether the indicator shows what the equipment is doing or what it has been commanded to do - Users can conduct a test of all indicators to identify failures - Where a control and its associated display element are not co-located, the control operates in a direction consistent with the movement of the nearest corresponding indicator or element on the display | NUREG-0700 NASA-STD-3001 IP |
|  | Visual alerts: - Flashing is only used to highlight items requiring urgent attention and is not used on items that must be studied in detail - The flashing area only covers a small part of the display - No more than two rates of flash are used (e.g. fast and slow) - Light indicators are not so bright as to create ‘dazzle’ or destroy operators’ dark adaptation ability (e.g. at night) | NUREG-0700 MIL-STD-1472H |
|  | Positioning: - Leave enough space between adjacent round indicator lights so a label can be placed next to each light - Each indicator is positioned or shielded so that bright ambient light will not cause the indicator to appear lit | MIL-STD-1472H |
|  | Luminance: - Indicators contrast with their background, e.g. through different luminance and colour - Luminance is sufficient to ensure recognition of indicators in the ambient lighting conditions - Under low-light or night-time conditions, luminance shall be controlled to avoid impairing the user’s dark adaptation - Increased luminance is reserved for higher-priority indications and is not used uniformly across all indicators - Indicative luminance ranges are: - Dark environments: ~1–10 cd/m² - Dim indoor: ~10–100 cd/m² - Typical indoor lighting: ~100–300 cd/m² - High ambient / outdoor: >300 cd/m² | BS EN ISO 9241 – Part 303 MIL-STD-1472H EEMUA 201 |

Table 13 provides guidance on size and colour coding of simple indicator lights to convey meaning.  Note that the use of colour can be culturally specific, and care must be taken to select colours that are meaningful in the culture of the operators.

Table 13: Simple indicator lights:  size and colour coding (MIL-STD-1472H)

| Simple indicator lights: size and colour coding |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size/type | Colour |  |  |  |  |
|  | Red | Yellow | Green | White | Blue |
| ≥25 mm (1 in) Flashing (3 to 5 Hz) | Emergency condition (impending personnel or equipment disaster) | N/A | N/A | N/A | N/A |
| ≥25 mm (1 in), Steady | Master summation* (system or subsystem) | Extreme caution (impending danger) | Master summation (system or subsystem) | N/A | N/A |
| ≤13 mm (0.5 in), Steady | Malfunction; action stopped; failure; stop action | Delay; check; recheck | Go ahead; in tolerance; acceptable; ready | Functional or physical position; action in progress | Advisory |

* a single overall indicator that summarises the status of multiple items within a system or subsystem

### Scales

The design of scales allows fast and accurate reading of critical measurements, reducing the risk of operators misreading or misinterpreting information required for their tasks.

Table 14 provide general design principles for the use of scales on HMIs.

Table 14: Design principles for scales

| # | Design Principle | Ref |
| --- | --- | --- |
|  | Numbers on scales are oriented upright | NUREG-0700 |
|  | The pointer on fixed scales extends from the right of vertical scales and from the bottom of horizontal scales | NUREG-0700 |
|  | The pointer extends to the shortest graduation marks without obscuring them | NUREG-0700 |
|  | Scales are not cluttered with more marks than necessary for precision | NUREG-0700 |

### System messages

This section specifies guidance for the wording, format, and consistency of messages used within the HMI, including normal, degraded, failure and emergency conditions, to ensure operators clearly understand system status and required actions, reducing the risk of misinterpretation or delayed response.

Table 15 provides the design principles for messaging.

Table 15: Design principles for messaging

| # | Design principle | Ref |
| --- | --- | --- |
|  | Structure: - Wording is concise and easily understood, with good syntax - Short, simple sentences are used with the main topic near the beginning of the sentence - Messages have consistent grammatical structure - Similar words used for different functions are clearly differentiated | NUREG-0700 BS EN ISO 9241-13:1998 BS IEC 61772:2013 |
|  | Phrasing: - Wording is familiar to operators and consistent with the task and operational guidance materials - Wording is consistent across tasks and systems - Different types of messages (e.g. advisory vs emergency) are standardised and distinct from each other - Messages are phrased in the active voice and highlight operator control rather than system control of the task - The system is not presented as a person - The item is described accurately | NUREG-0700 BS EN ISO 9241-13:1998 BS IEC 61772:2013 |
|  | Content: - The operator is informed of the type of error and its cause - Messages guide the operator on the next action(s) - Messages are positive statements emphasising what the operator should do, without being patronising - When an error occurs in a sequence of actions triggered by a single command, the operator is informed which actions were successfully completed by the system and which failed - The consequence of an action is described before the instructions on how to complete it - Similar words used for different functions are clearly differentiated | NUREG-0700 BS EN ISO 9241-13:1998 |
|  | Location: - Error messages appear in the location of the command or entry (without obscuring the operator’s view), or consistently in one location on the display - Users can move error messages around on the display | NUREG-0700 BS EN ISO 9241-13:1998 |
|  | Sequencing: - Messages are prioritised so that the highest-priority condition requiring operator action is clearly presented - Long lists of concurrent messages are avoided - Multiple errors are reported to the operator individually, rather than being grouped into one error message - Error messages close when the issue is resolved, or if the operator requests that they are removed - High priority messages and communications override low priority ones | NUREG-0700 BS EN ISO 9241-13:1998 BS IEC 61772:2013 IP |
|  | Context: - Areas on the display that relate to the message are highlighted - Error messages relating to the process in the plant are easily distinguished from error messages for the HMI itself - Users can request more detailed information or explanations when an error message is displayed, in online help | NUREG-0700 BS EN ISO 9241-13:1998 |
|  | Spoken messages: - Spoken messages are used sparingly - Spoken messages are short, simple and use formal language - Users can re-play spoken messages - Spoken messages are clearly audible and intelligible in the operational environment in which they are used | NUREG-0700 IP |

### Alarms and alerts

The presentation of alarms and alerts ensure operators can promptly detect, prioritise, and respond to abnormal or hazardous conditions, reducing the risk of missed, misunderstood or incorrectly prioritised warnings.

An alarm is a distinct sound or tone that demands immediate operator attention and action to address a system deviation, fault or hazard. Audible alarms are typically prioritised according to urgency and safety impact.

An alert is a system-generated message that draws operator attention to a condition or change in status that may require awareness or subsequent action, but not necessarily immediate intervention.

Designers should apply the principles set out in: Engineering Equipment and Materials Users’ Association (EEMUA) Publication 191 Alarm systems – A guide to design, management and procurement, EEMUA 201 Control rooms: A guide to their specification, design, commissioning and operation and IEC 63303 International Standard: Human machine interfaces for process automation systems.  These provide comprehensive, industry-recognised guidance on alarm philosophy, prioritisation and management to support effective operator response.

## Interaction methods and controls

This section defines principles for the design, identification, and operation of interaction methods, including mechanical or soft controls, within the HMI to ensure operators can reliably select and execute the intended actions, reducing the risk of incorrect, unintended or delayed control inputs.

### Control types

Control types described in this section are defined, with examples, in Table 16.

Table 16: Control type definitions

| # | Control type | Definition |
| --- | --- | --- |
|  | Soft | A control implemented within a graphical user interface, typically operated via a touchscreen, mouse, or similar input device. Soft controls are software-based and may change function, appearance, or availability depending on the system state or operating context Examples include: - A touchscreen button on a supervisory control and data acquisition (SCADA) or distributed control system (DCS) display used to start or stop a pump - A software slider or input field used to adjust a turbine setpoint or voltage limit - A screen-based valve open/close command in a control room HMI |
|  | Hard/mechanical | A physical control device with a fixed mechanical form that is directly manipulated by the operator, such as push buttons, switches, knobs, levers or handwheels. These controls provide tactile interaction and are typically used for frequently used, safety-critical or time-critical functions Examples include: - A physical push button on a control panel used to start or stop a generator - A rotary selector switch used to select operating mode (e.g. local/remote/maintenance) - A handwheel used to manually operate a valve |
|  | Multi-use or multi-functional | A control that performs different functions at different times, depending on the system mode, context or operator selection. The active function is clearly indicated to the operator to prevent ambiguity or incorrect operation Examples include: - A touchscreen control that performs different actions depending on the screen context, such as acknowledging alarms on one display and issuing commands on another - A shared keypad used to enter different types of process parameters depending on the active control screen - A combined push-and-rotate knob used to select and confirm a control setpoint (rotate to adjust value, push to confirm) - A joystick control used for both positioning equipment and confirming selections depending on the movement or press action |
|  | Dedicated | A control that performs a single, fixed function that does not change with system mode or context. Dedicated controls are typically used for important or frequently used functions where consistent and immediate access is required Examples include: - A hardwired emergency stop push button for plant shutdown - A dedicated trip reset button on a turbine control panel - A specific breaker open/close switch assigned to one piece of equipment in a substation control panel |

### Control design and functionality

Table 17 provides general principles for the design and functionality of all types of control.

Table 17: Design principles applicable to mechanical and soft controls

| # | Design principle | Ref |
| --- | --- | --- |
|  | Functional allocation: - Each control has a consistent function across operator tasks and all HMIs. It is clear which function a multi-use control is currently available for - Users are not required to switch frequently between controls for each action - Dedicated controls are provided for controls in constant use and emergency stops. If the dedicated control is a button, the operator only needs to press a single button. Otherwise, dedicated controls are minimised to avoid clutter - Controls with the same function are the same size. There are no more than three sizes of control if size is used to enable the operator to discriminate between controls | NUREG-0700 BS EN 61227:2016 |
|  | Location: - Controls are in a convenient place for the task they are used in - Controls with the same or similar function are in consistent locations across HMIs and panels - Controls do not limit freedom of movement for other tasks - Apart from controls that are required to be built in, control devices can be moved around the workstation by the operator - Controls that are located out of the operator’s line of sight have tactile features to distinguish them | NUREG-0700 NASA-STD-3001 BS EN ISO 11064-4 IP |
|  | Usability and reliability: - Controls are sufficiently accurate for the tasks they are used for - One-handed controls can be used either with the left or the right hand - Controls are rugged enough to withstand intended routine use, emergency use, and the environment - Controls are visible and operable in any personal protective equipment (PPE) required for the task - Control devices are stable during use; they do not slip or rock - Mechanical controls include stops at the ends of ranges of control positions | NUREG-0700 NASA-STD-3001 |
|  | Safeguards for accidental activation: - Controls can only be activated by intentional effort to avoid accidental activation or drifting - Emergency controls are protected against accidental activation, e.g. by a cover or collar - Single controls with multiple functions are not used where the operator is likely to get confused and make errors - Controls that are controlled by multiple operators have safeguards to prevent conflicting actions and make it clear to each operator who currently has control and the status of the system - Controls with hazardous or irreversible actions are easily distinguished by the operator | NUREG-0700 NASA-STD-3001 BS EN 61227:2016 IP |
|  | Status information and feedback: - The operator is informed if controls are locked out of use - The system gives visual, auditory, tactile and/or haptic feedback when control inputs are received and when the action has been executed by the system - Feedback is not distracting - The operator receives feedback on which function has been actioned to allow them to correct errors quickly - A warning is given before an operator can override safeguards - Commands with a direct impact on the facility have a validation step after selection before the command is actioned, apart from where a quick response is required - Controls that are located out of the operator’s line of sight have tactile features to distinguish them | NUREG-0700 BS EN ISO 9241-13:1998 NASA-STD-3001 BS EN ISO 11064-4 BS EN 61227:2016 IP |

### Control coding

Control coding refers to the use of visual, tactile, or spatial characteristics to distinguish different controls and communicate their function to the operator. The purpose is to help operators identify and operate the correct control quickly and accurately, reducing the risk of human error.

Table 18 compares different methods of control coding and their advantages to aid selection.

Table 18: Control coding:  advantages of different coding styles (MIL-STD-1472H)
(Ticks in the column for each type of coding indicate that they have the advantage described on the left.)

| Control coding: Advantages of different coding styles |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Advantage | Type of coding |  |  |  |  |  |
|  | Location | Shape | Size | Mode of operation | Labelling | Colour |
| Improves visual identification | ü | ü | ü |  | ü | ü |
| Improves non-visual identification (tactile and kinaesthetic) | ü | ü | ü | ü |  |  |
| Helps standardisation | ü | ü | ü | ü | ü | ü |
| Usable under low levels of illumination | ü | ü | ü | ü |  |  |
| May aid in identifying control position |  | ü |  | ü | ü | ü |
| Requires little training |  |  |  |  | ü |  |
| Requires minimal space |  |  |  |  |  | ü |
| Does not impact usability |  |  |  |  | ü | ü |
| More coding category options |  |  |  |  | ü |  |
| Usable with gloves | ü |  |  |  | ü | ü |

### Control separation

Separation means the minimum physical distance between rotary controls so that:

- The operator’s hand or fingers can comfortably grip and rotate the control.

- Adjacent controls are not accidentally moved.

- The correct control can be identified quickly.

Table 19 provides minimum edge-to-edge separation distances for mechanical controls.

Table 19: Minimum edge to edge separation distances for mechanical controls (MIL-STD-1472H)

| Minimum edge to edge separation distances for mechanical controls |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | Toggle switches | Push buttons* | Continuous rotary controls | Rotary selector switches | Discrete thumbwheel controls |
| Toggle switches | See Table 20 | 13 mm (0,5 in) | 19 mm (0,75 in) | 19 mm (0,75 in) | 13 mm (0,5 in) |
| Push buttons* | 13 mm (0.5 in) | See Table 20 | 13 mm (0,5 in) | 13 mm (0,5 in) | 13 mm (0,5 in) |
| Continuous rotary controls | 19 mm (0,75 in) | 13 mm (0,5 in) | See Table 20 | 25 mm (1 in) | 19 mm (0,75 in) |
| Rotary selector switches | 19 mm (0,75 in) | 13 mm (0,5 in) | 25 mm (1 in) | See Table 20 | 19 mm (0,75 in) |
| Discrete thumbwheel controls | 13 mm (0,5 in) | 13 mm (0,5 in) | 19 mm (0,75 in) | 19 mm (0,75 in) | See Table 20 |
| Notes: All values are for one handed operation. All values are for bare handed operation. *For push buttons not separated by barriers. |  |  |  |  |  |

Table 20 provides the recommended edge to edge control separation distances under different manual operation conditions.  Gloved hand measurements are provided where available. Otherwise, to adjust for a standard cotton flame-resistant anti-flash glove, add 5mm (0,2 in) to each dimension or 13mm (0,5 in) for thicker gloves (MIL-STD-1472H).

Table 20: Separation distances under different operation conditions for mechanical controls (MIL-STD-1472H)

| Control and operation type | Value type | Separation |  |
| --- | --- | --- | --- |
| Rotary selector switches |  |  |  |
| One hand | Minimum | 25 mm | 1 in |
|  | Preferred | 50 mm | 2 in |
| Two hand | Minimum | 75 mm | 3 in |
|  | Preferred | 125 mm | 5 in |
| Rocker switches |  |  |  |
| Bare hand | Minimum | 19 mm centre-centre | 0,75 in |
| Gloved hand | Minimum | 32 mm centre-centre | 1,125 in |
| Discrete thumbwheel controls |  |  |  |
| One hand | Minimum | 10 mm | 0,4 in |
| Rotary controls (knobs) |  |  |  |
| One hand | Minimum | 25 mm | 1 in |
|  | Optimum | 50 mm | 2 in |
| Two hand | Minimum | 50 mm | 2 in |
|  | Optimum | 125 mm | 5 in |
| Toggle switches |  |  |  |
| Single finger | Minimum | 19 mm | 0,75 in |
|  | Optimum | 50 mm | 2 in |
| Single finger sequential | Minimum | 13mm | 0,5 in |
|  | Optimum | 25 mm | 1 in |
| Different fingers simultaneous | Minimum | 16 mm | 0,625 in |
|  | Optimum | 19 mm | 0,75 in |
| Push buttons |  |  |  |
| Single finger Bare hand | Minimum | 13 mm | 0,5 in |
|  | Preferred | 50 mm | 2 in |
| Single finger Gloved hand | Minimum | 25 mm | 1 in |
| Single finger sequential | Minimum | 6 mm | 0,25 in |
|  | Preferred | 13 mm | 0,5 in |
| Different finger | Minimum | 6 mm | 0,25 in |
|  | Preferred | 13 mm | 0,5 in |
| Thumb/palm | Minimum | 25 mm | 1 in |
|  | Preferred | 150 mm | 6 in |
| Continuous Rotary Controls |  |  |  |
| One hand (Orientation A) Figure 11: Continuous rotary controls in orientation A | Minimum | 25 mm* | 1 in |
| One hand (Orientation B) Figure 12: Continuous rotary controls in orientation B | Minimum | 50 mm** | 2 in |
| Non-cranked handwheel controls |  |  |  |
| Configuration A – see Table 28 | N/A | 710 mm elbow-elbow clearance | 28 in |
| Configuration B – see Table 28 | N/A | 710 mm elbow-elbow clearance | 28 in |
| Configuration C – see Table 28 | N/A | 710 mm elbow clearance | 28 in |
|  |  | 100 to 150 mm overhead valve rim-to-rim clearance | 4 to 6 in |
| Lever controls |  |  |  |
| One hand | Minimum | 50 mm | 2 in |
|  | Preferred | 100 mm | 4 in |
| Two hand | Minimum | 75 mm | 3 in |
|  | Preferred | 125 mm | 5 in |
| Note: *Add 13 mm (0.5 in) for gloves **Add 25 mm (1 in) for gloves |  |  |  |

### Soft controls

Soft controls provide flexibility within the interface by allowing a single control to perform multiple functions; however, their design should ensure that the active function and method of operation are clear to the operator.

For allocating functions to soft controls, suitable consideration is needed to satisfy HFE requirements, for example: software switch selection time, human error rate in selecting the switches, or system response time.  This should be considered in the HMI design guide.

Table 21 provides design principles for soft controls.

Table 21: Design principles for soft controls

| # | Design principles | Ref |
| --- | --- | --- |
|  | Functional allocation: - Soft controls allow the operator to access individual components (e.g. plant item, equipment etc.) where required, and information is provided on the status of each component and its control relationship to other components - The excessive use of multiple modes (changes to the meaning or functioning of the control without changing how it looks) in soft control is avoided. When multiple modes are unavoidable, they are clearly indicated, so that the operator can easily determine the current mode | NUREG-0700 BS EN 61227:2016 |
|  | Cursors: - Cursors have distinctive visual features to ensure they are easily detectable by operators - Cursors are stable (not blinking) and do not disappear beyond display boundaries or out of sight - Travel distance across windows is minimised, i.e. the operator doesn’t need to move long distances with the cursor across multiple windows or displays - The operator can easily control the cursor to conduct all tasks. Cursor sensitivity can be adjusted by the operator - Location: - On opening a window, the cursor is easy to locate and in a convenient place - If the cursor has a home location this is consistent across displays - The cursor returns to a convenient location automatically for repetitive tasks (e.g. the input field) | NUREG-0700 BS EN ISO 9241-13:1998 BS EN 61227:2016 |
|  | Control selection time (the time between selection and acknowledgement to the operator) is appropriate for the task | BS EN 61227:2016 |

### Hard (mechanical) controls

Hard mechanical controls provide a reliable and direct means for operators to interact with equipment and systems. These controls support precise manual input and are often used where tactile feedback, robustness, or rapid operator response is required.

This section provides design guidance for commonly used mechanical control types, including rotary controls, thumb wheels, cranked handwheels and push buttons, ensuring they are designed and positioned to support safe, accurate and efficient operation.

#### Discrete rotary controls

Discrete rotary controls enable operators to select from a set of predefined options by rotating the control through clearly defined positions or detents. Table 22 provides dimensions and resistance for rotary controls.

Figure 13: Discrete rotary control

Table 22: Dimensions and resistance for discrete rotary controls (MIL-STD-1472H)

|  | Dimensions and resistance for discrete rotary controls |  |  |  |  |  |  |  |  |  | Illustration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Dimensions |  |  |  |  |  | Displacement* (A) |  | Resistance |  | Figure 14: Discrete rotary control dimensions and resistance |
|  | Length (L) |  | Width (W) |  | Height (H) |  |  |  |  |  |  |
| Minimum | 25 mm | 1 in | N/A |  | 16 mm | 0,625 in | 262 mrad | 15° | 115 mN.m | 1 in-lbf |  |
| Maximum | 100 mm | 4 in | 25 mm | 1 in | 75 mm | 3 in | 700 mrad | 40° | 680 mN.m | 6 in-lbf |  |
| Note | *Measurement given is for facilitating performance. Special engineering requirements or controls to be used tactually (‘blind’) may demand greater displacement |  |  |  |  |  |  |  |  |  |  |

#### Continuous adjustment rotary controls

Continuous adjustment rotary controls, also known as thumbwheels, enable operators to select from a set of predefined options by rotating the control through clearly defined positions or detents.

A continuous adjustment control is a type of rotary control designed to be operated with the thumb or finger that moves continually, or to defined start/stop positions only.

Table 23 provides dimensions and resistance for continuous adjustment rotary control.

Figure 15: Examples of continuous adjustment rotary controls

Table 23: Dimensions and resistance for continuous adjustment thumbwheel controls (MIL-STD-1472H)

| Dimensions and resistance for continuous adjustment thumbwheel controls |  |  |  |  |  | Illustrations |
| --- | --- | --- | --- | --- | --- | --- |
|  | Rim exposure (E) |  | Width (W) |  | Resistance |  | Figure 16: Dimensions and resistance for continuous adjustment thumbwheel controls |
| Minimum | 25 mm* | 1 in | 3 mm* | 0,125 in | No defined measure: should minimize effects of inadvertent input if operator is subject to motion |  |  |
| Maximum | 100 mm | 4 in | 23 mm | 0,875 in | 3,3 N | 12 oz |  |
| Note | *Preferred; some miniature applications may require less |  |  |  |  |  |  |

#### Knobs

Knobs are rotary control devices designed to be grasped and turned by the fingers or the whole hand to adjust a parameter. They are typically mounted on the front face of a panel and provide a simple means of controlling settings such as level, position or intensity.

Knobs may allow continuous adjustment (smooth rotation through a range) or discrete adjustment (rotation through defined steps or detents).

Figure 17: Example of a rotary knob control

Table 24: Dimensions for knobs (MIL-STD-1472H)

| Dimensions for knobs |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fingertip grasp (a) |  |  |  | Thumb and finger encircled (b) |  |  |  | Palm grasp (c) |  |  |  |
|  | Height (H) |  | Diameter (D) |  | Height (H) |  | Diameter (D) |  | Diameter (D) |  | Length (L) |  |
| Minimum | 13 mm | 0,5 in | 10 mm | 0,4 in | 13 mm | 0,5 in | 25 mm | 1 in | 38 mm | 1,5 in | 75 mm | 3 in |
| Maximum | 25 mm | 1 in | 100 mm | 4 in | 25 mm | 1 in | 75 mm | 3 in | 75 mm | 3 in | N/A |  |
| Illustration | Figure 18: Rotary knob control dimensions for fingertip grasp |  |  |  | Figure 19: Rotary knob control dimensions for thumb and finger encircled grasp |  |  |  | Figure 20: Rotary knob control dimensions for palm grasp |  |  |  |

Table 25: Torque for knobs (MIL-STD-1472H)

| Torque for knobs |  |  |  |
| --- | --- | --- | --- |
|  | ≤25 mm (1 in) diameter knobs |  | >25 mm (1 in) diameter knobs |  |
| Maximum | 32 mN.m | 4,5 in-ozf | 42 mN.m | 6 in-ozf |

A discrete adjustment thumbwheel control is a type of rotary control designed to be operated with the thumb or finger that moves through distinct detented positions, each corresponding to a specific predefined value or setting.

Table 26 provides dimensions for discrete adjustment thumb wheel controls.

Figure 21: Example of a discrete adjustment thumbwheel control

Table 26: Dimensions and resistance for discrete adjustment thumbwheel controls (MIL-STD-1472H)

| Dimensions and resistance for discrete adjustment thumbwheel controls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Diameter (D) |  | Trough distance (L) |  | Trough depth (H) |  | Width (W) |  | Resistance |  |
| Minimum | 29 mm | 1,125 in | 11 mm | 0,425 in | 3 mm | 0,125 in | 3 mm | 0,125 in | 1,7 N | 6 oz |
| Maximum | 75 mm | 3 in | 19 mm | 0,75 in | 6 mm | 0,25 in | N/A |  | 5,6 N | 20 oz |
| Illustration | Figure 22: Dimensions and resistance for discrete adjustment thumbwheel controls |  |  |  |  |  |  |  |  |  |

#### Cranked handwheel controls

A cranked handwheel control is a mechanical rotary control consisting of a handwheel with an offset crank handle that allows the operator to apply greater torque and make repeated rotations comfortably. These controls are typically used where larger adjustments or sustained manual operation are required.  Table 27 provides dimensions and turning radius.

Figure 23: Example of a cranked handwheel control

Table 27: Handle dimensions and turning radii for cranked handwheel controls (MIL-STD-1472H)

| Handle dimensions and turning radii for cranked handwheel controls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Load |  | Dimensions |  |  |  |  | Turning radius (R) |  |  |  |
|  |  |  | Length (L) |  | Diameter (D) |  | Rate below 100 RPM |  | Rate above 100 RPM |  |
| Light loads <22 N (5 lb); wrist and finger movement |  | Minimum | 25 mm | 1 in | 10 mm | 0,4 in | 38 mm | 1,5 in | 13 mm | 0,5 in |
|  |  | Preferred | 38 mm | 1,5 in | 13 mm | 0,5 in | 75 mm | 3 in | 65 mm | 2,5 in |
|  |  | Maximum | 75 mm | 3 in | 16 mm | 0,63 in | 125 mm | 5 in | 115 mm | 4,5 in |
| Heavy loads ≥22 N (5 lb); arm movement |  | Minimum | 75 mm | 3 in | 25 mm | 1 in | 190 mm | 7,5 in | 125 mm | 5 in |
|  |  | Preferred | 95 mm | 3,75 in | 25 mm | 1 in | N/A | N/A | N/A | N/A |
|  |  | Maximum | -- |  | 38 mm | 1,5 in | 510 mm | 20 in | 230 mm | 9 in |
| Illustration |  | Figure 24: Handle dimensions and turning radii for cranked handwheel controls |  |  |  |  |  |  |  |  |

#### Non-cranked handwheel controls

A non-cranked handwheel control is a mechanical rotary control consisting of a circular handwheel that is rotated directly by the operator’s hand around its rim. Unlike cranked handwheels, these controls do not include an offset handle and are typically used where lower torque is required or where space constraints limit the use of a crank.

Table 28 provides configurations, applications and dimensions for non-cranked handwheel controls.

Figure 25: Example of a non-cranked handwheel control

Table 28: Configurations, applications and dimensions for non-cranked handwheel controls (MIL-STD-1472H)

| Configurations, applications and dimensions for non-cranked handwheel controls |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # | Configuration | Example application | Dimensions |  |  |  |  |  |
|  |  |  | Diameter |  | Rim diameter |  | Minimum hand clearance (around rim) |  |
| A | Figure 26: Placement in relation to a display reference | Continuous adjustment for alternate slewing/precise positioning using display reference. Resistance low (e.g., below 110 N [25 lb]) | 200 to 510 mm | 8 to 20 in | 19 to 32 mm | 0,75 to 1,125 in | 75 mm | 3 in |
| B | Figure 27: Placement as a locking/unlocking mechanism | Continuous lock-unlock operation | 200 mm (8 in) for 22 N (5 lb) to 510 mm (20 in) for 155 N (35 lb) |  | 19 to 32 mm | 0,75 to 1,125 in | 75 mm | 3 in |
| C | Figure 28: High torque valves in various positions | High torque valves | Above standing surface: - 200 to 400 mm (8 to 16 in) For overhead: - 200 to 510 mm (8 to 20 in) For other position: - 300 to 1 520 mm (12 to 60 in) |  | 19 to 32 mm | 0,75 to 1,125 in | 75 mm | 3 in |

#### Push button controls (finger and hand operated)

A push button control is a finger- or hand-operated control that momentarily or continuously makes or breaks an electrical circuit when pressed. A push button control consists of a spring-loaded plunger within a casing that automatically returns to its original position when released. It provides a tactile response for reliable actuation.

Table 29 provides dimensions and resistance for push button controls.

Figure 29: Example of a push button

Table 29: Dimensions and resistance for push button controls (MIL-STD-1472H)

| Dimensions and resistance for push button controls |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Fingertip |  |  |  | Thumb |  |  |  | Palm |  |  |  |  | Resistance |  |  |  |  |  |
|  | Bare hand |  | Gloved hand |  | Bare hand |  | Gloved hand |  | Bare hand |  | Gloved hand |  |  | Single |  | Different fingers |  | Thumb/ palm |  |
| Minimum diameter | 10 mm | 0,4 in | 19 mm | 0,75 in | 19 mm | 0,75 in | 25 mm | 1 in | 40 mm | 1,6 in | 50 mm | 2 in |  | 2,8 N | 10 oz | 1,4 N | 5 oz | 2,8 N | 10 oz |
| Maximum diameter | 25 mm | 1 in | N/A |  | 25 mm (1 in) |  | N/A |  | 70 mm | 2,8 in | N/A |  |  | 11 N | 40 oz | 5,6 N | 20 oz | 23 N | 80 oz |
| Illustration | D Figure 30: Push button diameter |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

#### Toggle switch controls

A toggle switch control is a manually operated actuator that maintains its position to make or break an electrical circuit until it is switched again. It typically consists of a lever or rocker that moves between two or more positions, providing a stable on/off or multi-position control.

Table 30 provides dimensions and resistance for toggle switch controls.

Figure 31: Example of a toggle switch control

Table 30: Dimensions and resistance for toggle switch controls (MIL-STD-1472H)

| Dimensions and resistance for toggle switch controls |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Arm length (L) |  |  |  | Control tip diameter (D) |  | Resistance |  |  |  |
|  | Bare finger |  | Gloved hand |  |  |  | Small switch |  | Large switch |  |
| Minimum | 13 mm | 0,5 in | 38 mm | 1,5 in | 3 mm | 0,125 in | 2,8 N | 10 oz | 2,8 N | 10 oz |
| Maximum | 50 mm | 2 in | 50 mm | 2 in | 25 mm | 1 in | 4,5 N | 16 oz | 11 N | 40 oz |
| Illustration | Figure 32: Dimensions of a toggle switch |  |  |  |  |  |  |  |  |  |

#### Rocker switch controls

A rocker switch control is an actuator that rocks back and forth to make or break an electrical circuit, maintaining its position until switched again, commonly used for reliable on/off or multi-position control in various devices.

Table 31 provides dimensions and resistance for rocker switch controls.

Figure 33: Example of a rocker switch control

Table 31: Dimensions and resistance for rocker switch controls (MIL-STD-1472H)

| Dimensions and resistance for rocker switch controls |  |  |  |  |  | Illustration |
| --- | --- | --- | --- | --- | --- | --- |
|  | Dimensions |  |  |  | Resistance |  | Figure 34: Dimensions for rocker switch controls |
|  | Width (W) |  | Length (L) |  |  |  |  |
| Minimum | 6 mm | 0,25 in | 13 mm | 0,5 in | 2,8 N | 10 oz |  |
| Maximum | N/A |  | N/A |  | 11 N | (40 oz) |  |

#### Lever controls

Lever controls are actuating devices that either maintain position or return to an original position after being operated, providing precise mechanical control over connected systems. Levers transmit force through a pivoting arm.  Table 32 provides diameters and resistance for lever controls.

Figure 35: Example of a lever control

Table 32: Diameters and resistance for lever controls (MIL-STD-1472H)

| Diameters and resistance for lever controls |  |  |  |  |  |  |  |  |  |  |  | Illustration |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Diameter |  |  |  | Resistance |  |  |  |  |  |  |  | Figure 36: Diameters and operational directions for lever controls |
|  |  |  |  |  | Favourable biomechanical posture, back and forth (A d-1) |  |  |  | Cross-body or reaching posture, side-to-side (A d-2) |  |  |  |  |
|  | Finger grasp (D) |  | Hand grasp (D) |  | One hand |  | Two hand |  | One hand |  | Two hand |  |  |
| Minimum | 13 mm | 0,5 in | 38 mm | 1,5 in | 9 N | 2 lb | 9 N | 2 lb | 9 N | 2 lb | 9 N | 2 lb |  |
| Maximum | 38 mm | 1,5 in | 75 mm | 3 in | 135 N | 30 lb | 220 N | 50 lb | 90 N | 20 lb | 135 N | # b |  |

# ANNEX A GLOSSARIES

## A.1 Acronyms and abbreviations

%			percent

°			degrees

°C			degrees Celsius

BS			British Standard

cd/m²			candela per metre squared

CVD			colour vision deficiency

D			diameter

DCS			distributed control system

DITLO			day in the life of

e.g.			for example

EEMUA			Engineering and Equipment and Materials Users Association

EI			Energy Institute

H			height

HFE			human factors engineering

HMI			human machine interface

HSE			Health and Safety Executive

Ibf			pound-force

IEC			International Electrotechnical Commission

in			inches

in-ozf			inch-ounce-force

IOGP			International Association of Oil & Gas Producers

IP			industry practice

ISO			International Standards Organisation

L			length

MIL-STD		Military Standard

min			minutes (of arc)

mm			millimetre

mN.m			millinewton-metres

mrad			milliradian

N			Newtons

N/A			not applicable

NASA			National Aeronautics and Space Administration

N.m			Newton metre

NUREG			Nuclear Regulation

Oz/ozf			ounce (force)

PPE			personal protective equipment

RPM			revolutions per minute

SCADA			supervisory control and data acquisition

SI			International System of Units

US			United States

USNRC			United States Nuclear Regulatory Commission

VDU			visual display unit

W			width

W:H			width to height (ratio)

## A.2 Units of measurement

A2.1 Angle

| Degree (°) | A common unit for measuring angles. One degree represents 1/360 of a full circle. Degrees are widely used to describe general orientation or larger angles in geometry and diagrams |
| --- | --- |
| Arcminute (minute of arc, ‘, or min) | A smaller subdivision of a degree used for fine angular measurements. One arcminute equals 1/60 of a degree. It is commonly used to describe small visual angles or alignment tolerances where degrees would be too coarse |
| Milliradian (mrad) | A small angle equal one-thousandth of a radian. Milliradians are often used in engineering and optical calculations because they relate directly to geometry, allowing small angular changes to be easily converted into linear displacement over distance |

A2.2 Resistance

| Ounce-force (oz or ozf) | A small unit of force (1/16 of a pound-force) based on the weight of one ounce under normal Earth gravity. It is commonly used to describe light forces such as spring forces, switch actuation forces, or small mechanical loads |
| --- | --- |
| Pound-force (lb or lbf) | A unit of force equal to the weight of once pound under normal Earth gravity. It is widely used to describe moderate mechanical forces in equipment specifications, loads, and operating requirements |
| Newton (N) | The international systems of units (SI) for force. One newton is the force required to accelerate a mass of one kilogram by one metre per second squared. It is widely used in engineering and scientific calculations to describe loads and mechanical forces |

A.2.3 Torque

| Inch-ounce-force (in-ozf) | A unit of torque representing the turning effect produced by a force of one ounce-force acting at a distance of one inch from a pivot point. It is typically used for small torques in precision mechanisms and small fasteners |
| --- | --- |
| Inch-pound-force (in-lbf) | A unit of torque representing the turning effect produced by a force of one pound-force acting at a distance of one inch from a pivot point. It is commonly used for specifying tightening torques and mechanical rotational loads |
| Newton-metre (N.m) | The standard SI unit of torque. It represents the turning effect produced when a force of one newton acts at a distance of one metre from a pivot point. It is commonly used for specifying tightening torques and rotational forces in mechanical systems |

# ANNEX B REFERENCES

EEMUA

EEMUA 191 Alarm systems – A guide to design, management and procurement (4th edition).

EEMUA 201 Control rooms – A guide to their specification, design, commissioning and operation (3rd edition).

Energy Institute (EI)

EI 454 Human factors engineering in projects (2nd edition).

Health and Safety Executive (HSE)

Website, Control room design. Accessed 15 August 2025. Available at: https://www.hse.gov.uk/comah/sragtech/techmeascontrol.htm

International Electrotechnical Commission (IEC)

IEC 63303 Human-machine interfaces for process automation systems (1st edition).

BS EN 61227:2016 Nuclear power plants – Control rooms – Operator controls.

BS IEC 61772:2013 Nuclear power plants – Control rooms – Application of visual display units (VDUs).

International Organization for Standardization (ISO)

BS EN ISO 9241-13:1998 Ergonomic requirements for office work with visual display terminals – Part 13: User guidance.

BS EN ISO 9241-303:2011 Ergonomics of human–system interaction – Part 303: Requirements for electronic visual displays.

BS EN ISO 11064-4:2013 Ergonomic design of control centres – Part 4: Layout and dimensions of workstations.

BS EN ISO 11064-5:2008 Ergonomic design of control centres – Part 5: Displays and controls.

ISO 9355-2:1999 Ergonomic requirements for the design of displays and control actuators, Part 2: Displays (withdrawn)

International Society of Automation (ISA)

ISA-101.01-2015 Human machine interfaces for process automation systems.

NASA

NASA-STD-3001 NASA spaceflight human system standard (3rd edition).

US Department of Defense

MIL-STD-1472H Design criteria standard – Human engineering.

US Nuclear Regulatory Commission (US NRC)

NUREG-0700 Human system interface design review guidelines (3rd edition).

The specified W:H ratios are advisory. Font legibility is influenced by typeface design, particularly the characteristics of lowercase letters, and the size and position of the ascenders and descenders which may affect perceived character size and readability. Therefore, the selected font and size should be validated through usability testing to ensure text remains clearly legible under the intended operating conditions.

The physical dimensions of a point vary depending on the font type, rendering method, and display characteristics. Where precise legibility requirements are critical, character size is verified using actual rendered dimensions (e.g. height in mm).